AZURE NAT GATEWAY FOR PREDICTABLE OUTBOUND CONNECTIVITY

Project Code: 24CC3046-P035

Domain: Cloud Networking

1. ABSTRACT

Modern cloud applications require reliable, secure, and predictable outbound Internet connectivity. Virtual Machines deployed in private Azure subnets may need to communicate with external services without being assigned public IP addresses directly. Azure NAT Gateway provides a managed solution for this requirement by performing Source Network Address Translation (SNAT) and providing a predictable public IP address for outbound traffic.

This project implements an Azure NAT Gateway with a private Ubuntu Virtual Machine deployed inside an Azure Virtual Network. The Virtual Machine does not have a public IP address and uses a Standard NAT Gateway for outbound Internet connectivity. A Standard Public IP address is associated with the NAT Gateway to provide a predictable public source IP.

A Network Security Group (NSG) is additionally configured to control outbound traffic. Five approved destination IP addresses are allowed, while a deny rule blocks other Internet destinations. This provides an additional layer of outbound traffic control while maintaining the NAT Gateway's primary function of predictable outbound connectivity.

The implementation was verified using Linux networking commands and Internet connectivity tests. The VM private IP was verified as 10.0.1.4, while the NAT Gateway uses the public IP 4.224.18.198. Before applying the restrictive outbound allowlist, the command curl -4 ifconfig.me returned 4.224.18.198, confirming that the VM's outbound traffic was being translated through the NAT Gateway.

2. INTRODUCTION

Cloud-based applications frequently require outbound Internet access to communicate with APIs, software repositories, monitoring services, and other external resources. Providing a public IP address directly to every Virtual Machine is not always desirable because it increases exposure and makes centralized outbound IP management more difficult.

Azure NAT Gateway provides centralized and managed outbound connectivity for resources deployed in a subnet. It allows private resources to access the Internet while using one or more configured public IP addresses for outbound connections.

In this project, an Ubuntu Virtual Machine is deployed without a public IP address inside a private subnet. The subnet is associated with an Azure NAT Gateway. When the VM initiates an outbound connection, NAT Gateway performs Source Network Address Translation and uses its public IP as the source address.

An additional Network Security Group is used to restrict outbound traffic to approved destination IP addresses. This creates a controlled egress architecture combining private networking, predictable outbound addressing, and outbound traffic filtering.

3. PROBLEM STATEMENT

Virtual Machines deployed in private cloud networks require a reliable method to access the Internet without necessarily exposing the Virtual Machines through public IP addresses.

The major challenges include:

- Providing Internet access to private Virtual Machines.

- Maintaining a predictable outbound public IP address.

- Avoiding the need for a public IP on every Virtual Machine.

- Controlling which external destinations a workload can access.

- Handling large numbers of outbound connections efficiently.

- Reducing the risk associated with SNAT port exhaustion.

- Managing cloud networking costs.

Therefore, this project implements Azure NAT Gateway to provide predictable outbound connectivity and an NSG to control outbound access.

4. OBJECTIVES

The main objectives of this project are:

- To provide predictable outbound Internet connectivity for a private Azure Virtual Machine.

- To deploy a Virtual Machine without a public IP address.

- To implement Source Network Address Translation using Azure NAT Gateway.

- To provide a stable public IP address for outbound traffic.

- To restrict outbound traffic using an NSG allowlist.

- To block unauthorized Internet destinations.

- To provide scalable SNAT capacity and reduce the risk of SNAT port exhaustion.

- To understand the cost considerations of Azure networking resources.

5. TECHNOLOGIES USED

- Microsoft Azure

- Azure Virtual Network

- Azure Subnet

- Azure NAT Gateway

- Azure Standard Public IP

- Azure Network Security Group

- Azure Virtual Machine

- Ubuntu Server 24.04 LTS

- Azure Run Command

- Linux

- Bash

- curl

- ip

- ss

- TCP/IP

- SNAT

6. SYSTEM ARCHITECTURE

The implemented architecture consists of a private Virtual Machine, private subnet, Network Security Group, NAT Gateway, and Standard Public IP.

INTERNET

↑

│

Public IP

4.224.18.198

↑

│

SNAT

↑

NAT Gateway

natgw-project

↑

│

Network Security Group

vm-nat-test-nsg

│            │

Allowed IPs      Other IPs

ALLOW           DENY

│             │

└──────┬──────┘

↑

Subnet

10.0.1.0/24

↑

│

Ubuntu VM

vm-nat-test

10.0.1.4

7. AZURE RESOURCE CONFIGURATION

7.1 Resource Group

Name: rg-nat-project

The Resource Group contains the resources used for the project.

7.2 Virtual Network

Name: vnet-nat-project

Address Space: 10.0.0.0/16

The Virtual Network provides the private networking environment.

7.3 Subnet

Name: default

Address Range: 10.0.1.0/24

The subnet contains the private Virtual Machine.

7.4 Virtual Machine

Name: vm-nat-test

Operating System: Ubuntu Server 24.04 LTS

VM Size: Standard B2as_v2

Region: Central India

Private IP: 10.0.1.4

The VM does not have a public IP address.

7.5 NAT Gateway

Name: natgw-project

SKU: Standard

Region: Central India

TCP Idle Timeout: 4 minutes

Associated Subnet: vnet-nat-project/default

7.6 Public IP

Name: pip-nat-project

SKU: Standard

Public IP: 4.224.18.198

This public IP is used by the NAT Gateway for outbound traffic.

7.7 Network Security Group

Name: vm-nat-test-nsg

The NSG is associated with the Virtual Machine's network interface and controls outbound traffic.

8. NAT GATEWAY WORKING

The Virtual Machine has the private IP address:

10.0.1.4

When the VM sends traffic to the Internet, the traffic passes through the NAT Gateway.

Before SNAT:

Source: 10.0.1.4

Destination: Internet

The NAT Gateway performs Source Network Address Translation.

After SNAT:

Source: 4.224.18.198

Destination: Internet

Therefore, external Internet services see the NAT Gateway public IP instead of the VM's private IP.

The overall flow is:

VM 10.0.1.4

↓

Private Subnet

↓

NAT Gateway

↓

SNAT

↓

4.224.18.198

↓

Internet

9. NETWORK SECURITY GROUP CONFIGURATION

The existing NSG vm-nat-test-nsg was configured with five outbound allow rules.

The approved destination IP addresses are:

1.1.1.1

8.8.8.8

1.0.0.1

8.8.4.4

9.9.9.9

The rules are configured with priorities:

| Priority | Rule Name | Destination | Action |
| --- | --- | --- | --- |
| 100 | Allow-IP-01 | 1.1.1.1 | Allow |
| 110 | Allow-IP-02 | 8.8.8.8 | Allow |
| 120 | Allow-IP-03 | 1.0.0.1 | Allow |
| 130 | Allow-IP-04 | 8.8.4.4 | Allow |
| 140 | Allow-IP-05 | 9.9.9.9 | Allow |
| 4000 | Deny-All-Internet | Internet | Deny |

The custom allow rules have higher priority than the deny rule.

If the destination matches one of the approved IP addresses, the connection is allowed.

If the destination does not match any of the approved IP addresses, the Deny-All-Internet rule blocks the traffic.

10. IMPLEMENTATION PROCESS

Step 1: Create the Resource Group

A Resource Group named rg-nat-project was created in Azure to organize all project resources.

Step 2: Create the Virtual Network

A Virtual Network named vnet-nat-project was created with the address space 10.0.0.0/16.

Step 3: Create the Subnet

A subnet named default was created with the address range 10.0.1.0/24.

Step 4: Deploy the Virtual Machine

An Ubuntu Server 24.04 LTS Virtual Machine named vm-nat-test was deployed inside the subnet.

The VM was configured without a public IP address.

Its private IP address was verified as 10.0.1.4.

Step 5: Create the NAT Gateway

A Standard NAT Gateway named natgw-project was created in Central India.

The NAT Gateway was associated with the default subnet.

Step 6: Configure the Public IP

A Standard Public IP named pip-nat-project was associated with the NAT Gateway.

The public IP address was:

4.224.18.198

Step 7: Configure the NSG

The existing Network Security Group vm-nat-test-nsg was used to implement outbound filtering.

Five destination IP addresses were allowed and a deny rule was configured for other Internet traffic.

11. IMPLEMENTATION VERIFICATION

11.1 Verify Private IP

The following command was executed from the VM using Azure Run Command:

ip a

The VM showed:

10.0.1.4/24

This confirmed that the VM was using a private IP address.

11.2 Verify Routing

The following command was executed:

ip route

The VM showed a default route through the subnet gateway.

This confirmed that the VM had a route for outbound traffic.

11.3 Verify Internet Connectivity

Before implementing the restrictive outbound allowlist, the following command was executed:

curl -I https://www.google.com

The command returned an HTTP success response:

HTTP/2 200

This confirmed that the private VM had outbound Internet connectivity.

11.4 Verify NAT Gateway Public IP

The following command was executed:

curl -4 ifconfig.me

The result was:

4.224.18.198

This matched the public IP configured on the NAT Gateway.

Therefore, the VM's outbound traffic was successfully translated through the NAT Gateway.

12. OUTBOUND ALLOWLIST TESTING

After implementing the NSG allowlist, approved destinations were permitted while unapproved destinations were blocked.

The logical behavior is:

Approved IP

↓

NSG Allow Rule

↓

ALLOW

↓

NAT Gateway

↓

Internet

For an unapproved destination:

Unapproved IP

↓

No matching Allow Rule

↓

Deny-All-Internet

↓

BLOCK

This demonstrates controlled outbound Internet access.

13. GOOGLE AND IFCONFIG.ME TESTING

Before implementing the restrictive allowlist, the following commands were used to verify outbound Internet connectivity:

curl -4 ifconfig.me

and:

curl -I https://www.google.com

The NAT IP test returned:

4.224.18.198

After implementing the strict destination allowlist, these commands may be blocked if the destination IP addresses used by ifconfig.me or Google are not included in the allowlist.

This behavior is expected because the NSG filters traffic based on destination IP addresses.

Large Internet services such as Google may use multiple IP addresses, so allowing only one IP does not guarantee access to the complete service.

14. CONCURRENT OUTBOUND CONNECTION TEST

A controlled concurrent outbound traffic test was performed using:

for i in {1..200}; do

curl -s --max-time 20 https://httpbin.org/delay/10 > /dev/null &

done

sleep 2

echo "Active TCP connections:"

ss -ant | wc -l

echo "NAT public IP:"

curl -4 ifconfig.me

wait

The purpose of this test was to generate multiple outbound requests concurrently.

The for loop attempts to create 200 requests.

The & symbol runs each request in the background.

The sleep 2 command provides time for the requests to start.

The ss -ant command displays TCP socket information.

The curl -4 ifconfig.me command was used to check the public IPv4 address.

The wait command waits for the background requests to finish.

The TCP socket count is a point-in-time observation and does not prove that exactly 200 simultaneous TCP connections were established.

15. SNAT PORT EXHAUSTION

SNAT port exhaustion can occur when an application creates a very large number of outbound connections and available source ports become exhausted.

Azure NAT Gateway provides scalable SNAT capacity for outbound connections and therefore reduces the risk of SNAT port exhaustion.

The project does not claim to completely eliminate SNAT port exhaustion.

The correct conclusion is:

Azure NAT Gateway provides scalable SNAT capacity and reduces the risk of SNAT port exhaustion for workloads generating large numbers of outbound connections.

Azure Monitor NAT Gateway metrics can be used to observe SNAT connection activity and investigate failed connections.

16. SECURITY BENEFITS

The implemented architecture provides the following security benefits:

Private VM

The VM does not have a public IP address and therefore is not directly exposed to unsolicited Internet traffic.

Controlled Outbound Access

The NSG allows only approved destination IP addresses.

Predictable Public IP

The NAT Gateway provides a known public IP address for outbound traffic.

Centralized Connectivity

Outbound connectivity is centralized through the NAT Gateway instead of assigning public IP addresses to individual VMs.

Reduced Exposure

The private VM remains behind the Azure networking infrastructure while still being able to initiate permitted outbound connections.

17. ADVANTAGES

The project provides the following advantages:

- Predictable outbound public IP address.

- Private VM deployment.

- No public IP required on the VM.

- Centralized outbound connectivity.

- Destination-based outbound filtering.

- Controlled Internet access.

- Scalable SNAT architecture.

- Easier external IP allowlisting.

- Improved control over outbound communication.

- Suitable for cloud applications requiring predictable egress.

18. LIMITATIONS

IP-Based Filtering

IP-based allowlisting can become difficult when external services use multiple or changing IP addresses.

NAT Gateway Cost

NAT Gateway has resource-hour and data-processing charges.

Dynamic Internet Services

Services such as Google may use multiple IP addresses, meaning that allowing a single IP does not guarantee access to the entire service.

Outbound Focus

NAT Gateway is designed primarily for outbound connectivity and does not provide general unsolicited inbound access to the VM.

NSG and NAT Gateway Have Different Roles

The NSG controls traffic, while NAT Gateway performs outbound SNAT. An NSG does not replace NAT Gateway.

19. COST OPTIMIZATION

Azure networking resources should be monitored to avoid unnecessary costs.

The following cost-saving practices can be followed:

- Deallocate the VM when it is not being used.

- Delete the NAT Gateway when the project is completed.

- Retain the Public IP if it needs to be reused.

- Monitor Azure for Students credit usage.

- Remove unused Azure resources.

- Avoid unnecessary monitoring features that generate additional charges.

20. FUTURE ENHANCEMENTS

The project can be extended with:

Azure Monitor

NAT Gateway metrics can be used to monitor SNAT connections, packets, bytes, dropped packets, and datapath availability.

Alerts

Alerts can be configured for unusual connection behavior and failed SNAT connections.

Multiple Virtual Machines

Additional VMs can be added to the subnet to demonstrate centralized outbound connectivity.

Infrastructure as Code

The environment can be automated using Azure CLI, Bicep, ARM templates, or Terraform.

Advanced Egress Filtering

More advanced Azure networking services can be evaluated when filtering needs to be based on applications, domains, or more complex security policies rather than fixed IP addresses.

21. TEST CASES

| Test Case | Expected Result | Status |
| --- | --- | --- |
| Verify VM private IP | 10.0.1.4 | PASS |
| Verify routing | Default route available | PASS |
| Test Internet connectivity | Internet accessible before restrictive filtering | PASS |
| Test NAT public IP | 4.224.18.198 | PASS |
| Test approved destination | Connection allowed | PASS |
| Test unapproved destination | Connection blocked | PASS |
| Verify NSG rules | Allowlist and deny rule configured | PASS |
| Generate concurrent outbound requests | Requests generated | PASS |
| Verify NAT Gateway configuration | Static public IP configured | PASS |

22. REAL-WORLD APPLICATION

This architecture can be used in real-world cloud environments where private workloads need controlled outbound Internet access.

For example, a company may have a private application server that needs to communicate with approved external APIs.

The architecture can be:

Private Application Server

↓

NSG

↓

Approved External Services

↓

NAT Gateway

↓

Static Public IP

↓

Internet

External services can allowlist the NAT Gateway public IP instead of the private IP of individual servers.

23. CONCLUSION

This project successfully demonstrates Azure NAT Gateway for predictable outbound connectivity from a private Virtual Machine.

An Ubuntu Virtual Machine was deployed inside a private Azure Virtual Network without a public IP address. The VM was assigned the private IP address 10.0.1.4.

A Standard Azure NAT Gateway named natgw-project was associated with the subnet and configured with the Standard Public IP 4.224.18.198.

Before applying the restrictive outbound allowlist, the command:

curl -4 ifconfig.me

returned:

4.224.18.198

This confirmed that outbound traffic from the private VM was being translated through the NAT Gateway.

An additional Network Security Group was configured with five approved destination IP addresses and a deny rule for other Internet destinations. This provided controlled outbound access while maintaining the predictable outbound IP functionality of the NAT Gateway.

The project demonstrates the combination of private networking, Source Network Address Translation, predictable outbound IP addressing, outbound traffic filtering, and scalable SNAT infrastructure.

Overall, the implementation provides a practical Azure networking architecture for private workloads that require secure, controlled, and predictable outbound Internet connectivity.

Top of Form

Bottom of Form